# Milestone 4: Development Phase (Coding and Testing)

Implementation Plan

The Kanban process will be used for management of this project. Progress will be tracked through the use of a task board on Trello, where each task can be moved between columns (Backlog, To Do, In Progress, Complete). The following table represents current progress.

| Use Case /User Story | List of Development Tasks | Estimate (hrs.) | Actual (hrs.) | Percent Complete |
| --- | --- | --- | --- | --- |
| Preparation & Setup | PCB layout/design | 20 | 12 | 100% |
|  | PCB Manufacturing | 0 (vendor) | 0 | 100% |
|  | PCB Assembly/Testing | 20 | 22 | 100% |
| Insert & Removal | PoC Test Fixture | 30 | 31 | 100% |
|  | Test Fixture final design/printing | 5 | 0 | 100% |
| Firmware loading | Import existing project code | 5 | - | - |
|  | Customization to imported code | 10 | - | - |
|  | UI development | 15 | - | - |
| Hardware Verification |  |  |  |  |
| - Turn on power | PoC for power management | 10 | 3 | 100% |
|  | Firmware to turn on module power | 5 | 8 | 100% |
| - Validate current consumption | PoC for current measurement | 10 | 6 | 100% |
|  | Firmware to test current values and return results | 5 | - | - |
| - Diagnostic mode | Firmware to set I/O pins | 5 | - | - |
| - Test CV inputs | PWM-to-analog POC | 10 | 5 | 100% |
|  | Firmware to set CV values and read results | 5 | - | - |
| - Test OPT pins | Firmware to set I/O pins and read results | 5 | - | - |
| - Test sampling rates | Firmware to set I/O pins and read results | 5 | - | - |
| - Test tap temp | Firmware to send timed pulses and read results | 5 | - | - |
| - Test audio | Audio multiplexer PoC | 10 | 7 | 100% |
|  | Firmware to stimulate all audio inputs and observe all audio outputs | 5 | - | - |
| - Test memory | Firmware to set module mode, send inputs and observe delayed output | 5 | - | - |
| - Turn off power | Firmware to turn module power off | 5 | 1 | 100% |
| Results Management | Develop PC code to process results | 20 | - | - |
| Stock Management | Integrate software with enterprise server/database | 10 | - | - |
| User Control/Notification | Develop UI that allows users to start, stop and view test results | 5 | 8 | 75% |

Percent of User Stories complete for this iteration: 100%

Percent of User Stories complete for entire project: 50%

Mapping of Functional Requirements

The following table provides a mapping between use cases/user stories, test cases and the architecture design document.

| (Use Case/User Story) | (Architecture Plan) Page or section where designed | (Code Module) Class and Method where implemented | (Test Case) Test Case number from the Test Plan |
| --- | --- | --- | --- |
| Preparation & Setup | Pg 4 (Hardware) | (N/A) | (N/A) |
| Insert & Removal | Pg 4 (Hardware) | (N/A) | 5 |
| Firmware loading | Pg 5 (Software) | TBD | 11 |
| Turn on power | Pg 8 (Pseudocode) | CT3680TestPlatform.ino: void relayPower(bool turnOn) | 2 |
| Validate current consumption | Pg 8 (Pseudocode) | TBD | 1 |
| Diagnostic mode | Pg 8 (Pseudocode) | TBD | 16 |
| Test CV inputs | Pg 8 (Pseudocode) | TBD | 17 |
| Test OPT pins | Pg 8 (Pseudocode) | TBD | 18 |
| Test sampling rates | Pg 8 (Pseudocode) | TBD | 19 |
| Test tap temp | Pg 8 (Pseudocode) | TBD | 20 |
| Test audio | Pg 8 (Pseudocode) | TBD | 21 |
| Test memory | Pg 8 (Pseudocode) | TBD | 22 |
| Turn off power | Pg 8 (Pseudocode) | CT3680TestPlatform.ino: void relayPower(bool turnOn) | 2 |
| Results Management | Pg 5 (Software), Pg 13 (Configuration), Pg 22, Sequence Diagram), Pg 23 (Server API’s) | TBD | 8, 25 |
| Stock Management | Pg 5 (Software), Pg 22 (Sequence Diagram), Pg 23 (Server API’s) | TBD | 8, 24 |
| User Control/Notification | Pg 5 (Software/User Interface Mockups), Pg 20-22 (User Interface Diagrams) | CT3680TestPlatformGUI.html | 10 |

Code Listing

## Microcontroller Code:

GitHub: https://github.com/cabintech/CT3680TestPlatformFirmware.git

Description:

C code: CT3680TestPlatformGUI.ino – contains functions for each testing procedure such as power on/off, audio multiplexing, etc. These are not all coded yet, but will be added during the next iteration of development.

## PC Code:

GitHub: https://github.com/cabintech/CT3680TestPlatformUI.git

Description:

Servlet: TestCom.java – tests the communication between the PC and the microcontroller by sending power on/off commands

HTML: CT3680TestPlatformGUI.html – contains all HTML code for the user interface. Currently contains the script as well, although that will later be moved to its own file

CSS: testPlatform.css – contains all styling rules for the user interface

## PCB Design:

This diagram shows the electrical routing between the various components on the PCB.

## Schematics - Base Fixture:

This diagram shows the circuits connected to each of the module pins.

## Schematics - LCD Display:

This diagram shows the circuitry associated with the LCD display.

## Schematics – Main Page:

This is the primary page that shows how each of the components on the PCB is electrically connected to each other.

## Schematics – Power Control:

This diagram shows the circuitry associated with the power relay, which turns power on and off to the module.

## Schematics – PWM to Analog Converter:

These resistor/capacitor pairs convert PWM signals from the microcontroller to analog signals

## Test Fixture

This 3D image shows the current design of the test fixture. This is the third and final iteration.

Test Plan and Test Cases
System Tests

| Id: 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Current Detection Circuit |  |  |  |  |  |  |  |
| System: Hardware |  |  |  |  |  |  |  |
| Test Objective: Verify that current detection system works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Create short circuit and verify that the module fails |  | Two voltage readings pre/post resistor | Out of range (fail) |  | (test not yet run) |
| 2 |  | Create open circuit and verify that the module fails |  |  |  |  | (test not yet run) |
| 3 |  | Test a known good module and verify that it passes |  |  | In range (pass) |  | (test not yet run) |

| Id: 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Power Control |  |  |  |  |  |  |  |
| System: Hardware |  |  |  |  |  |  |  |
| Test Objective: Verify that power control system works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Turn power on |  | Two voltage readings at module’s power and ground pins | Module has power |  | Pass (module has power) |
| 2 |  | Verify that PCB has power |  |  |  |  |  |
| 3 |  | Turn power off |  |  | Module does not have power |  | Pass (module does not have power) |
| 4 |  | Verify that PCB does not have power |  |  |  |  |  |

| Id: 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Audio Multiplexing |  |  |  |  |  |  |  |
| System: Hardware |  |  |  |  |  |  |  |
| Test Objective: Verify that audio multiplexing system works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps (For each of the 4 input channels) |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Connect input source |  | Listening for audio on each channel |  |  |  |
| 2 |  | Route signal to each of the 4 outputs (1 at a time) |  |  |  |  |  |
| 3 |  | Verify audio on output channel |  |  | Clear audio present |  | (test not yet run) |
| 4 |  | Verify absence of audio on other channels |  |  | No audio/static noise |  | (test not yet run) |

| Id: 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: PWM to Analog Conversion |  |  |  |  |  |  |  |
| System: Hardware |  |  |  |  |  |  |  |
| Test Objective: Verify that PCB circuitry correctly converts PWM signals to analog |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Produce PWM signals via the microcontroller |  | Electrical signal readings observed with oscilloscope | Smooth wave form showing on oscilloscope (pass) |  | (test not yet run) |
| 2 |  | Measure signal between capacitor and module input pin to verify a smooth, consistent signal |  |  |  |  |  |

| Id: 5 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Module Insert/Removal |  |  |  |  |  |  |  |
| System: Hardware |  |  |  |  |  |  |  |
| Test Objective: Verify that module can be easily inserted and removed without causing damage |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Insert module into test fixture |  | Time and ease of insert and removal, pin connections | Module insert/removal is quick and easy, and does not damage any of the components (module, pins, fixture, etc.) |  | Pass |
| 2 |  | Verify that it makes reliable connections to all pins |  |  |  |  |  |
| 3 |  | Remove the module from the test fixture |  |  |  |  |  |
| 4 |  | Repeat 3 times and check for damage to pins, module and fixture |  |  |  |  |  |

| Id: 6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Correct pass/fail categorization of modules |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that good modules pass and bad modules are correctly detected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Insert a known good module |  | Test cycle results | Module categorized as good (pass) |  | (test not yet run) |
| 2 |  | Run hardware testing cycle |  |  |  |  |  |
| 3 |  | Replace good module with known bad module |  |  | Module categorized as bad (fail) |  | (test not yet run) |
| 4 |  | Run hardware testing cycle |  |  |  |  |  |

| Id: 7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Insert new module into database |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that new modules are successfully inserted into the database |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Execute code to insert a new module into the database |  | Database query results | New module exists in database (pass) |  | (test not yet run) |
| 2 |  | Query the database for the new module and verify that it was inserted |  |  |  |  |  |

| Id: 8 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Correct pass/fail handling of modules |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that the correct actions are performed in the database based on test results |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Insert a passing module into the database |  | Two database observations (before/after) | Registration table is updated with the new module’s serial number/results and stock count is incremented by one |  | (test not yet run) |
| 2 |  | Observe changes in stock count and registration |  |  |  |  |  |
| 3 |  | Insert a failing module into the database |  |  | Registration table is updated but stock count remains the same |  | (test not yet run) |
| 4 |  | Observe changes in stock count and registration |  |  |  |  |  |

| Id: 9 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Database handling of re-tested modules |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that re-tested modules are correctly updated in the database |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Attempt to insert an already registered module into the database |  | Two database observations (before/after) | Insert fails due to duplicate serial number |  | (test not yet run) |
| 2 |  | Attempt to update the existing record with most recent results |  |  | Record is updated with most recent results and date/timestamp |  | (test not yet run) |

| Id: 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: UI Handling of re-tested modules |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that re-test verification screen is displayed and that the appropriate response is taken |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Insert a registered module into the test fixture and start test cycle |  | UI Display | UI returns to start page |  | (test not yet run) |
| 2 |  | Verify UI informs the user that the module is already registered, click cancel on verification page |  |  |  |  |  |
| 3 |  | Verify that UI returns to start page, click start again |  |  |  |  |  |
| 4 |  | Click proceed on re-test verification page |  |  | Testing cycle is initialized |  | (test not yet run) |
| 5 |  | Verify that testing continues |  |  |  |  |  |

| Id: 11 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Module Firmware Update |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that the firmware update feature works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Insert module |  |  | Firmware is successfully updated (pass) |  | (test not yet run) |
| 2 |  | Run update process and observe results |  |  |  |  |  |
| 3 |  | Remove module |  |  | Firmware update fails |  | (test not yet run) |
| 4 |  | Run update process again and observe results |  |  |  |  |  |

| Id: 12 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Network Failure Handling |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that the software does not break when network connection fails |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Disconnect PC from internet |  | Observe how the program handles the failure | Firmware update fails |  | (test not yet run) |
| 2 |  | Insert module and attempt to update firmware |  |  |  |  |  |

| Id: 13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Software/Firmware integration |  |  |  |  |  |  |  |
| System: Software/Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that PC application can connect, send and receive data from microcontroller |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Run PC/microcontroller code |  | Application Log | Log verifies communication between PC and microcontroller |  | (no log in place yet other than the console, but commands over the COM port are successfully processed) |
| 2 |  | Observe PC application log |  |  |  |  |  |

| Id: 14 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: PC error handling for unexpected/missing response |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that the software does not break when microcontroller fails to respond as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Start test cycle |  | PC response to faulty communication with microcontroller | PC handles the failure gracefully (displays error message, returns to start screen, tries to reestablish connection, etc.) |  | (test not yet run) |
| 2 |  | Turn off power supply |  |  |  |  |  |
| 3 |  | Observe PC response |  |  |  |  |  |

| Id: 15 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Cancel Test |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that testing cycle is abandoned if the user clicks cancel |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Start test cycle |  | UI interaction and observation | Upon clicking cancel, test procedures are halted and the UI returns to the start screen |  | (test not yet run) |
| 2 |  | Click cancel from the progress screen |  |  |  |  |  |
| 3 |  | Start a new test cycle |  |  | New cycle starts successfully and performs all tests |  | (test not yet run) |
| 4 |  | Verify that all tests are correctly performed |  |  |  |  |  |

Unit Tests

| Id: 16 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 1 (Diagnostic Testing Mode) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that a module can be put in diagnostic testing mode |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 17 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 2 (Control Voltage Inputs) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that all 6 control voltage inputs work as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 18 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 3 (Digital Option Inputs) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that all 5 digital option inputs work as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 19 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 4 (Module Firmware Programs) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that all 16 module firmware programs work as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 20 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 5 (Sampling Rates) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that all four sampling rates work as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 21 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 6 (Tap Tempo) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that tap tempo works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 22 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 7 (Audio Passthrough) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that audio passthrough works as expected on all 4 channels |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 23 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 8 (Internal Memory) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that internal memory works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 24 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Enterprise API (Stock Management System) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify correct integration with Stock Management System |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a test harness for PC/Server interaction |  | Observation of results | PC integrates smoothly with Stock Management System |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |

| Id: 25 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Enterprise API (Registration) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that internal memory works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a test harness for PC/Server interaction |  | Observation of results | PC integrates smoothly with Registration |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |

Acceptance Testing

Completion of system testing will constitute a successful acceptance test

Application Demonstration

The following video demonstrates the PCB design as well as the user interface, and demonstrates a working tech stack including UI, servlet and microcontroller.

https://youtu.be/GmREGDvxj6E