# Milestone 5: Development Phase (Coding and Testing)

Implementation Plan

The Kanban process will be used for management of this project. Progress will be tracked through the use of a task board on Trello, where each task can be moved between columns (Backlog, To Do, In Progress, Complete). The following table represents current progress.

| Use Case /User Story | List of Development Tasks | Estimate (hrs.) | Actual (hrs.) | Percent Complete |
| --- | --- | --- | --- | --- |
| Preparation & Setup | PCB layout/design | 20 | 12 | 100% |
|  | PCB Manufacturing | 0 (vendor) | 0 | 100% |
|  | PCB Assembly/Testing | 20 | 22 | 100% |
| Insert & Removal | PoC Test Fixture | 30 | 31 | 100% |
|  | Test Fixture final design/printing | 5 | 0 | 100% |
| Firmware loading | Import existing project code | 5 | - | - |
|  | Customization to imported code | 10 | - | - |
|  | UI development | 15 | - | - |
| Hardware Verification |  |  |  |  |
| - Turn on power | PoC for power management | 10 | 3 | 100% |
|  | Firmware to turn on module power | 5 | 8 | 100% |
| - Validate current consumption | PoC for current measurement | 10 | 6 | 100% |
|  | Firmware to test current values and return results | 5 | 5 | 100% |
| - Diagnostic mode | Firmware to set I/O pins | 5 | 6 | 100% |
| - Test CV inputs | PWM-to-analog POC | 10 | 5 | 100% |
|  | Firmware to set CV values and read results | 5 | 3 | 100% |
| - Test OPT pins | Firmware to set I/O pins and read results | 5 | 4 | 100% |
| - Test sampling rates | Firmware to set I/O pins and read results | 5 | 6 | 100% |
| - Test tap temp | Firmware to send timed pulses and read results | 5 | 4 | 100% |
| - Test audio | Audio multiplexer PoC | 10 | 7 | 100% |
|  | Firmware to stimulate all audio inputs and observe all audio outputs | 5 | - | - |
| - Test memory | Firmware to set module mode, send inputs and observe delayed output | 5 | - | - |
| - Turn off power | Firmware to turn module power off | 5 | 1 | 100% |
| Results Management | Develop PC code to process results | 20 | 10 | 50% |
| Stock Management | Integrate software with enterprise server/database | 10 | - | - |
| User Control/Notification | Develop UI that allows users to start, stop and view test results | 5 | 8 | 75% |

Percent of User Stories complete for this iteration: 100%

Percent of User Stories complete for entire project: 72%

Mapping of Functional Requirements

The following table provides a mapping between use cases/user stories, test cases and the architecture design document.

| (Use Case/User Story) | (Architecture Plan) Page or section where designed | (Code Module) Class and Method where implemented | (Test Case) Test Case number from the Test Plan |
| --- | --- | --- | --- |
| Preparation & Setup | Pg 4 (Hardware) | (N/A) | (N/A) |
| Insert & Removal | Pg 4 (Hardware) | (N/A) | 5 |
| Firmware loading | Pg 5 (Software) | TBD | 11 |
| Turn on power | Pg 8 (Pseudocode) | Util.cpp: void relayPowerOn() | 2 |
| Validate current consumption | Pg 8 (Pseudocode) | Util.cpp: void relayPowerOn() | 1 |
| Diagnostic mode | Pg 8 (Pseudocode) | Verify.cpp verifyDiagStream() | 16 |
| Test CV inputs | Pg 8 (Pseudocode) | Verify.cpp verifyVCSettings() | 17 |
| Test OPT pins | Pg 8 (Pseudocode) | Verify.cpp verifyOptionSwitches() | 18 |
| Test sampling rates | Pg 8 (Pseudocode) | Verify.cpp verifySamplingRates() | 19 |
| Test tap temp | Pg 8 (Pseudocode) | Verify.cpp verifyTapTempo() | 20 |
| Test audio | Pg 8 (Pseudocode) | Verify.cpp verifyAudioPassthrough() {not yet implemented} | 21 |
| Test memory | Pg 8 (Pseudocode) | Verify.cpp verifyMemory() {not yet implemented} | 22 |
| Turn off power | Pg 8 (Pseudocode) | Util.cpp: void relayPowerOff() | 2 |
| Results Management | Pg 5 (Software), Pg 13 (Configuration), Pg 22, Sequence Diagram), Pg 23 (Server API’s) | TBD | 8, 25 |
| Stock Management | Pg 5 (Software), Pg 22 (Sequence Diagram), Pg 23 (Server API’s) | TBD | 8, 24 |
| User Control/Notification | Pg 5 (Software/User Interface Mockups), Pg 20-22 (User Interface Diagrams) | CT3680TestPlatformGUI.html | 10 |

Code Listing

## Microcontroller Code:

GitHub: https://github.com/cabintech/CT3680TestPlatformFirmware.git

Description:

C code: CT3680TestPlatformGUI.ino – contains the main loop() and setup() functions to control the program.

CT3680TestPlatformGUI.h/Util.h/Verify.h: define all public method signatures, constants, etc. to be used throughout the program.

Util.cpp: provides all utility functions needed to build up higher level tests, such as powerOn/Off, readDiagnosticStream, etc.

Verify.cpp: holds the code for each of the hardware verification tests

## PC Code:

GitHub: https://github.com/cabintech/CT3680TestPlatformUI.git

Description:

Servlet: TestCom.java – tests the communication between the PC and the microcontroller by sending power on/off commands

HTML: CT3680TestPlatformGUI.html – contains all HTML code for the user interface. Currently contains the script as well, although that will later be moved to its own file

CSS: testPlatform.css – contains all styling rules for the user interface

## PCB Design:

This diagram shows the electrical routing between the various components on the PCB.

## Schematics - Base Fixture:

This diagram shows the circuits connected to each of the module pins.

## Schematics - LCD Display:

This diagram shows the circuitry associated with the LCD display.

## Schematics – Main Page:

This is the primary page that shows how each of the components on the PCB is electrically connected to each other.

## Schematics – Power Control:

This diagram shows the circuitry associated with the power relay, which turns power on and off to the module.

## Schematics – PWM to Analog Converter:

These resistor/capacitor pairs convert PWM signals from the microcontroller to analog signals

## Test Fixture

This 3D image shows the current design of the test fixture. This is the third and final iteration.

Test Plan and Test Cases
System Tests

| Id: 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Current Detection Circuit |  |  |  |  |  |  |  |
| System: Hardware |  |  |  |  |  |  |  |
| Test Objective: Verify that current detection system works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Create short circuit and verify that the module fails |  | Two voltage readings pre/post resistor | Out of range (fail) |  | (test not yet run) |
| 2 |  | Create open circuit and verify that the module fails |  |  |  |  | (test not yet run) |
| 3 |  | Test a known good module and verify that it passes |  |  | In range (pass) |  | (test not yet run) |

| Id: 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Power Control |  |  |  |  |  |  |  |
| System: Hardware |  |  |  |  |  |  |  |
| Test Objective: Verify that power control system works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Turn power on |  | Two voltage readings at module’s power and ground pins | Module has power |  | Pass (module has power) |
| 2 |  | Verify that PCB has power |  |  |  |  |  |
| 3 |  | Turn power off |  |  | Module does not have power |  | Pass (module does not have power) |
| 4 |  | Verify that PCB does not have power |  |  |  |  |  |

| Id: 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Audio Multiplexing |  |  |  |  |  |  |  |
| System: Hardware |  |  |  |  |  |  |  |
| Test Objective: Verify that audio multiplexing system works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps (For each of the 4 input channels) |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Connect input source |  | Listening for audio on each channel |  |  |  |
| 2 |  | Route signal to each of the 4 outputs (1 at a time) |  |  |  |  |  |
| 3 |  | Verify audio on output channel |  |  | Clear audio present |  | (test not yet run) |
| 4 |  | Verify absence of audio on other channels |  |  | No audio/static noise |  | (test not yet run) |

| Id: 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: PWM to Analog Conversion |  |  |  |  |  |  |  |
| System: Hardware |  |  |  |  |  |  |  |
| Test Objective: Verify that PCB circuitry correctly converts PWM signals to analog |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Produce PWM signals via the microcontroller |  | Electrical signal readings observed with oscilloscope | Smooth wave form showing on oscilloscope (pass) |  | (test not yet run) |
| 2 |  | Measure signal between capacitor and module input pin to verify a smooth, consistent signal |  |  |  |  |  |

| Id: 5 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Module Insert/Removal |  |  |  |  |  |  |  |
| System: Hardware |  |  |  |  |  |  |  |
| Test Objective: Verify that module can be easily inserted and removed without causing damage |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Insert module into test fixture |  | Time and ease of insert and removal, pin connections | Module insert/removal is quick and easy, and does not damage any of the components (module, pins, fixture, etc.) |  | Pass |
| 2 |  | Verify that it makes reliable connections to all pins |  |  |  |  |  |
| 3 |  | Remove the module from the test fixture |  |  |  |  |  |
| 4 |  | Repeat 3 times and check for damage to pins, module and fixture |  |  |  |  |  |

| Id: 6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Correct pass/fail categorization of modules |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that good modules pass and bad modules are correctly detected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Insert a known good module |  | Test cycle results | Module categorized as good (pass) |  | (test not yet run) |
| 2 |  | Run hardware testing cycle |  |  |  |  |  |
| 3 |  | Replace good module with known bad module |  |  | Module categorized as bad (fail) |  | (test not yet run) |
| 4 |  | Run hardware testing cycle |  |  |  |  |  |

| Id: 7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Insert new module into database |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that new modules are successfully inserted into the database |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Execute code to insert a new module into the database |  | Database query results | New module exists in database (pass) |  | (test not yet run) |
| 2 |  | Query the database for the new module and verify that it was inserted |  |  |  |  |  |

| Id: 8 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Correct pass/fail handling of modules |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that the correct actions are performed in the database based on test results |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Insert a passing module into the database |  | Two database observations (before/after) | Registration table is updated with the new module’s serial number/results and stock count is incremented by one |  | (test not yet run) |
| 2 |  | Observe changes in stock count and registration |  |  |  |  |  |
| 3 |  | Insert a failing module into the database |  |  | Registration table is updated but stock count remains the same |  | (test not yet run) |
| 4 |  | Observe changes in stock count and registration |  |  |  |  |  |

| Id: 9 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Database handling of re-tested modules |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that re-tested modules are correctly updated in the database |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Attempt to insert an already registered module into the database |  | Two database observations (before/after) | Insert fails due to duplicate serial number |  | (test not yet run) |
| 2 |  | Attempt to update the existing record with most recent results |  |  | Record is updated with most recent results and date/timestamp |  | (test not yet run) |

| Id: 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: UI Handling of re-tested modules |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that re-test verification screen is displayed and that the appropriate response is taken |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Insert a registered module into the test fixture and start test cycle |  | UI Display | UI returns to start page |  | (test not yet run) |
| 2 |  | Verify UI informs the user that the module is already registered, click cancel on verification page |  |  |  |  |  |
| 3 |  | Verify that UI returns to start page, click start again |  |  |  |  |  |
| 4 |  | Click proceed on re-test verification page |  |  | Testing cycle is initialized |  | (test not yet run) |
| 5 |  | Verify that testing continues |  |  |  |  |  |

| Id: 11 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Module Firmware Update |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that the firmware update feature works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Insert module |  |  | Firmware is successfully updated (pass) |  | (test not yet run) |
| 2 |  | Run update process and observe results |  |  |  |  |  |
| 3 |  | Remove module |  |  | Firmware update fails |  | (test not yet run) |
| 4 |  | Run update process again and observe results |  |  |  |  |  |

| Id: 12 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Network Failure Handling |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that the software does not break when network connection fails |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Disconnect PC from internet |  | Observe how the program handles the failure | Firmware update fails |  | (test not yet run) |
| 2 |  | Insert module and attempt to update firmware |  |  |  |  |  |

| Id: 13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Software/Firmware integration |  |  |  |  |  |  |  |
| System: Software/Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that PC application can connect, send and receive data from microcontroller |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Run PC/microcontroller code |  | Application Log | Log verifies communication between PC and microcontroller |  | (no log in place yet other than the console, but commands over the COM port are successfully processed) |
| 2 |  | Observe PC application log |  |  |  |  |  |

| Id: 14 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: PC error handling for unexpected/missing response |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that the software does not break when microcontroller fails to respond as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Start test cycle |  | PC response to faulty communication with microcontroller | PC handles the failure gracefully (displays error message, returns to start screen, tries to reestablish connection, etc.) |  | (test not yet run) |
| 2 |  | Turn off power supply |  |  |  |  |  |
| 3 |  | Observe PC response |  |  |  |  |  |

| Id: 15 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Cancel Test |  |  |  |  |  |  |  |
| System: Software |  |  |  |  |  |  |  |
| Test Objective: Verify that testing cycle is abandoned if the user clicks cancel |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Start test cycle |  | UI interaction and observation | Upon clicking cancel, test procedures are halted and the UI returns to the start screen |  | (test not yet run) |
| 2 |  | Click cancel from the progress screen |  |  |  |  |  |
| 3 |  | Start a new test cycle |  |  | New cycle starts successfully and performs all tests |  | (test not yet run) |
| 4 |  | Verify that all tests are correctly performed |  |  |  |  |  |

Unit Tests

| Id: 16 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 1 (Diagnostic Testing Mode) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that a module can be put in diagnostic testing mode |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | Hardware functions correctly and passes verification |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 17 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 2 (Control Voltage Inputs) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that all 6 control voltage inputs work as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | Testing has been run using some starter code but is not yet fully implemented |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 18 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 3 (Digital Option Inputs) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that all 5 digital option inputs work as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | Hardware functions correctly and passes verification |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 19 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 4 (Module Firmware Programs) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that all 16 module firmware programs work as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | Hardware functions correctly and passes verification |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 20 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 5 (Sampling Rates) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that all four sampling rates work as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | Hardware functions correctly and passes verification |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 21 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 6 (Tap Tempo) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that tap tempo works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | Hardware functions correctly and passes verification |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 22 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 7 (Audio Passthrough) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that audio passthrough works as expected on all 4 channels |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 23 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Hardware Verification Step 8 (Internal Memory) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that internal memory works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a firmware test harness to run this step of hardware verification |  | Observation of results before and after creating the fault | Hardware functions correctly (pass) |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |
| 3 |  | Create hardware fault |  |  | Hardware does not function correctly (fail) |  | (test not yet run) |
| 4 |  | Re-run test harness and observe results |  |  |  |  |  |

| Id: 24 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Enterprise API (Stock Management System) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify correct integration with Stock Management System |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a test harness for PC/Server interaction |  | Observation of results | PC integrates smoothly with Stock Management System |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |

| Id: 25 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Name: Unit test of Enterprise API (Registration) |  |  |  |  |  |  |  |
| System: Firmware |  |  |  |  |  |  |  |
| Test Objective: Verify that internal memory works as expected |  |  |  |  |  |  |  |
| Step |  | Test Steps |  | Test Data | Expected Results (Pass/Fail) |  | Actual Results |
| 1 |  | Construct a test harness for PC/Server interaction |  | Observation of results | PC integrates smoothly with Registration |  | (test not yet run) |
| 2 |  | Run test harness and observe results |  |  |  |  |  |

Acceptance Testing

Completion of system testing will constitute a successful acceptance test

Application Demonstration

The following video demonstrates a functioning sequence of tests, showing the results of each test being sent back to the PC over the COM port. The testing sequence includes powerOn, verifyDiagnosticStream, verifyOptionSwitches, verifyProgramSelection, verifySamplingRates, verifyTapTempo and powerOff.

https://youtu.be/dcvhrOSyPiM